Trolley Transverse Arm Replacement Process Procedures

Introduction

IST Engineering S.A has been requested by Piraeus Port Authority S.A. to issue explanatory procedures regarding the replacement process for the existing Trolley Transverse Arm (Figure No.1).

Following the FE Analysis for the existing Transverse Arm we arrive into the decision to design and study a new one for the same use completely replaceable with the existing one. The Transverse Arm is mounted on the Trolley Bogie of Rail Mounted Gantry Cranes (RMG), which are operating at the Port of Piraeus Container Terminal I.

Figure 1  Existing Transverse Arm mounted on the Trolley Bogie

Figure 2.  New design for the Transverse Arm.

Issued by:

Date: 22. January 2021

Anastasios Adamopoulos I.S.T. S.A.

Dipl. Mechanical Engineer

Approvals and version history

| REVISION | DATE | DESCRIPTION OF CHANGE | APPROVED BY |
| --- | --- | --- | --- |
| A | 05/01/2021 | Preliminary version | Adamopoulos An. |
| B | 22/01/2021 | Final version | Adamopoulos An. |

Contents

Abstract	4

Safety specification：	8

1st Step – Manufacturing of Transverse Arm.	8

2nd Step – Disassembly of the existing Transverse Arm.	8

3rd Step – Assembly of the new Transverse Arm.	9

Annex A	12

1.	SURFACE PREPARATION	12

2.	PAINTING PROCESS	12

3.	Tightening torques	13

4.	Welding Assembly drawing	14

## Abstract

The scope of this document is to describe all actions, works and assembly process for the “Trolley Transverse Arm ’’ replacement procedure.

On each RMG Trolley there are Four (4) Arms that hold the Gear Reducers, Brakes and Electrical Motors of the ‘’Driving’’ wheels with the Trolley Bogie (See Fig.No.3).

Transverse Arm

Silent Block

Gear Reducer

Electrical Motor

Brake

Figure 3. Transverse Arm mounted with Silent Block – Gear Reducer- Brake & Electrical Motor.

During the ordinary inspections from PPA maintenance Technical Team they found in the majority of the Arms severe cracks at the welding joints and immediately they repair temporarily all defected areas with welding reinforcements. (See below Fig. No. 4)

These problems cause a lot of delays in the operation of the Terminal Container I of PPA.

Welded plates

Welded Reinforcement

Figure 4. Repaired Transverse Arm.

Additionally we found that the Transverse Arm was not manufactured according to OEM manufacturing Drawing No. 12834-5612. Instead of square pipe 120X8, they used welded plates. (See Fig. 4)

In order to solve the issue, we decided to replace the excisting Arm with a new design with less welded parts, more robust, easy to manufacture and at the same time replaceable with the old one.

Please, refer to the below figures No. 5 & Νο.6 which indicates the “New Design Transverse Arm’’ mounted on the Trolley Bogie.

Figure 5. New Transverse Arm mounted on the Trolley Bogie.

Plates (10mm Thick.)

Plate (15mm Thick.)

Bottom Plate (15mm Thick.)

Stiffeners
(10mm Thick.)

Interface Plate (15mm Thick.)

Square Hollow Section 120x120x10 mm

Figure 6: Trolley Transverse Arm New Design

The new design consists of a single square hollow section with dimensions of 120x120x10 mm. In addition, four plates and two stiffeners are supporting the hollow section as presented in Figure 6

## Safety specification：

Following the Greek Safety Legislation only qualified-certified, trained, and designated person may carry out all necessary operations.

Note:  The welding operations must be carried out by certified personnel and the NDT (Non Destructive Tests) by a qualified company and certified by the Hellenic Accreditation System (ESYD).

## 1st Step – Manufacturing of Transverse Arm.

Please proceed with the manufacturing of the Transverse Arm According to IST S.A. manufacturing Drawing No.IST-A300-5612.

- Assembly and welding process please refer to annex Welding Assembly Drawing No. IST-A300-5612.

- Carry out the MT inspection around the welding areas to ensure that everything is correct. After MT completed you can proceed with the surface treatment and painting.

- For the Surface Treatment please refer to Annex point 1.

- For the coating procedure please refer to annex point 2 and table 1.

## 2nd Step – Disassembly of the existing Transverse Arm.

For the disassembly, please follow the instructions described on OEM’s maintenance manual.

Important Note

Before proceeding with the assembly of the new Transverse Arm you must carry out the following actions:

- Trolley bogie must be ready from their repair procedure.

- Trolley wheels must be aligned according to Std. FEM 1.001 3d Edition Booklet 8.

## 3rd Step – Assembly of the new Transverse Arm.

From the OEM drawing No. 12834-5600 the Center Line of the Transverse Arm must be perpendicular to the Center Line of the Trolley Rails (See below Figure No.6).

Intersection of the two center lines

Figure 7.  Transverse Arm Center Line perpendicular to Trolley Rails Center Line.

From the visual inspection on the RMG’s we noticed that it is not applicable in the existing situation.

In order to assembly the new Arm in the position according to OEM drawing No. 12834-5600 we decided to proceed as follows:

- Tap all the existing holes (X2 Ø20 H7 & X4 Ø22 H7) of the Arm Basement.

- Grinding the Trolley Basement Surface and touch up with paint.

- Place and assembly the new Arm with the Silent Block.

Note: It is recommended to replace all silent blocks with new ones.

- Mark up the new holes position.

- Drill on site the Trolley Basement.

- Mount the two Parallel pins (DWR No.  IST-P309-5612 - Figure No.8) and place the four bolts M 20X60 with the tab washer. Tight with a torque wrench the self-locking nuts M20 with a torque of 410 Nm using split lock washers. (See Figure No. 9).

(Assumption: Friction Coefficient 0, 14)

(See Appendix Table 2)  Tightening Forces in Nm According to:

EN 13001-3-1:2012+a2:2018 Cranes General Design Part3-1: Limit States and proof competence of steel structure.

- At the end of the assembly process secure the Parallel pins with Split pins DIN 94 (See Fig. No.8)

Parallel pins

Split pins DIN 94

Spring Split Lock Washers DIN127 HDG

DIN 127

Self-locking hexagon nuts DIN 982 HDG

DIN 127

Bolts M20Χ 60 8.8 DIN 933 HDG

Figure 8.  Parallel pins -Bolds – nuts-Washers and Split pins assembly.

Washer with two tabs DIN 463 HDG

DIN 127

Figure 9.  Tab Washers Assembly

Deliverables to PPA after the completion of the repair works for each RMG:

- ALL certificates from the supplier of the Steel Plates, welding materials and steel parts for each RMG separately.

- Welder’s certificate in force.

- ALL results from NDT’s for each RMG separately.

## Annex A

## SURFACE PREPARATION

The painting of the main steel structure will be executed after all oils, humidity and residues have  previously been removed, and after it has been subjected to SA 2 ½ sandblasting according to Swedish regulations SIS 055900 or ISO 8501-1/1998, in accordance with the paint manufactures recommendations. Where shot blasting is not practical, chipping, hammering, scrapping and wire brushing may be employed subject to approval of PPA. Oils and greases shall be removed by suitable solvents.

The following systems will be adopted:

- Washing (water cleaning).

- Mechanical or manual preparation.

- Preparation by sandblasting.

## PAINTING PROCESS

| Paint type | Dry film thickness (microns) | Number of coats | Color |
| --- | --- | --- | --- |
| First coat: Zinc epoxy primer | 65 | 1 |  |
| Second coat: Epoxy primer | 60 | 1 |  |
| Third coat: Epoxy primer | 60 | 1 |  |
| Fourth coat: Polyurethane, semigloss | 60 | 1 | RAL 9010 (White) |
| Total cycle thickness | 245 |  |  |

Table 1.

## Tightening torques

Table 2. EN 13001-3-1:2012+a2:2018 Cranes General Design Part3-1: Limit States and proof competence of steel structure.

## Welding Assembly drawing