REPAIR WORKS ON GANGWAYS IN PERAMA , NEW PORT OF DRAPETSONA

AND PORT OF PPC ( TOTAL 17 PCS )

A. PERAMA AREA ( 6 PCS )

001. GANGWAY No 35 – 25 m LENGTH

Welding of 34 cracks , 50 mm length each , on wood’s steel stays ( steel angle 50 X 50 X 5 ) .

Replacement of 12 steel angles , 40 X 40 X 4 , 8 pcs 1300 mm , 1 pc 1150 mm & 3 pcs 1200 mm from the truss .

Replacement of 2 handrail stanchions  , D 2” X 1100 mm & welding one crack .

Securing the woods by new galvanized cup head square neck bolts D 8 X 120 & D 8 X 70 with washers & nuts .

Manufacture & installation of one transportation wheel , D 250 X 50 , from steel with the same dimensions as the three existed ones ( with ball bearings ) .

Replacement of ball bearings of the other three and any other additional repair work .

Manufacture & replacement of the roller shaft , D 40 X 1200 mm , with new steel bushes .

Repair of the roller cover .

002. GANGWAY No 130 – 30 m LENGTH

Alignment of left handrail , length 3 m .

Welding of 10 cracks , on the base of the stanchions .

Manufacture & replacement of the roller shaft , D 40 X 1200 mm , with new steel bushes .

Inspection and restore the correct operation of the 4 transportation wheels .

003. GANGWAY No 230 – 30 m LENGTH

Replacement of 7 wooden steps , from pine , 5 X 7 X 85 cm , with new bolts , washers and nuts .

Alignment of left and right handrails , length 5 m per handrail .

Manufacture & replacement of the roller shaft , D 40 X 1200 mm , with new steel bushes .

Welding of 10 cracks , on the base of the stanchions .

Cut , repair & rewelding of the left hanging plate .

Inspection and restore the correct operation of  the 4 transportation wheels .

004. GANGWAY No 330 – 30 m LENGTH

Welding of 8 cracks , on the base of the stanchions .

Alignment of left handrail , length 5 m .

Manufacture & installation of one transportation wheel shaft , D 30 X 14 & one transportation wheel ,

D 205 X 60 , from steel , with the same dimensions as the three existed ones ( with ball bearings ) .

Replacement of ball bearings of the other three and any other additional repair work .

005. GANGWAY No 37 – 25 m LENGTH

Welding of 40 cracks , 50 mm length each , on wood’s steel stays ( steel angle 50 X 50 X 5 ) .

Replacement of 12 steel angles , 40 X 40 X 4 X 1300 mm of the truss .

Welding 10 cracks of the stanchion bases .

Manufacture & replacement of the roller shaft , D 40 X 1200 mm , with new steel bushes and restore the correct operation of the cover .

Inspection and restore the correct operation of  the 4 transportation wheels .

006. GANGWAY No 33 – 20 m LENGTH

Welding of 45 cracks , 50 mm length each , on wood’s steel stays ( steel angle 50 X 50 X 5 ) .

Replacement of 12 steel angles , 40 X 40 X 4 X 1300 mm , of the truss .

Welding 12 cracks of stanchion bases .

Manufacture & installation two transportation wheels , D 270 X 70 , from steel , with the same dimensions as the two existed ones , with new ball bearings & new shaft 40 X 40 X 1300 .

Inspection and restore the correct operation of  the 2 transportation wheels & roller .

B. NEW PORT OF DRAPETSONA ( 9 PCS )

001. GANGWAY No 46 – 25 m LENGTH

Replacement all the woods and steps as the old ones .

The total width of wood is 90 cm  , the steps are 63 pcs , 5 X 7 X 85 cm , from pine .

Welding of 40 cracks , 50 mm length each , on wood’s steel stays ( steel angle 50 X 50 X 5 ) .

Welding 8 cracks of stanchion bases .

Alignment of left handrail , length 5 m .

Replacement of bronze bushes , D 32 X D 60 X 50 mm of 4 transportation wheels .

Restore the correct operation of the roller & cover

002. GANGWAY No 66 – 25 m LENGTH

Replacement all the woods and steps as the old ones .

The total width of wood is 90 cm  , the steps are 63 pcs , 5 X 7 X 85 cm , from pine .

Welding of 30 cracks , 50 mm length each , on wood’s steel stays ( steel angle 50 X 50 X 5 ) .

Welding 6 cracks of stanchion bases & replacement 6 stanchions , D 2” X 1000 .

Replacement of bronze bushes , D 32 X D 60 X 50 mm of 4 transportation wheels .

Manufacture & replacement of the roller shaft , D 46 X 1150 , with new steel bushes & restore the correct operation of the roller cover .

Replacement of 10 steel angles , 40 X 40 X 4 X 1300 mm , of the truss .

003. GANGWAY No 115 – 15 m LENGTH

Welding 10 cracks of stanchion bases of handrails .

Replacement of 2 steel angles , 40 X 40 X 4 X 1500 mm , of the truss .

Cutting off the 2 transportation wheels , D 270 X 50 mm , from the roller side and rewelding them at a  new position on truss’ edge .

Restore the correct operation of them .

Welding of the roller cover .

Manufacture & replacement of the roller shaft , D 45 X 1150 mm , with new steel bushes .

004. GANGWAY No 415 – 15 m LENGTH

Replacement all the woods and steps as the old ones .

The total width of wood is 90 cm  , the steps are 38 pcs , 5 X 7 X 85 cm , from pine .

Cutting off the 2 transportation wheels , D 300 X 60 mm , from the roller side and rewelding them at new position at the truss’ edge , replacement of their shaft , D 30 X 150 with new bronze bushes .

Restore the correct operation of the roller and its cover .

Welding 12 cracks of stanchion bases .

005. GANGWAY No 36 – 25 m LENGTH

Replacement all the woods and steps as the old ones .

The total width of wood is 90 cm  , the steps are 63 pcs , 5 X 7 X 85 cm , from pine .

Manufacture & replacement of the roller shaft , D 40 X 1150 mm , with new steel bushes .

Restore the correct operation of the roller cover .

Alignment of left & right handrails , 5 m length of each handrail .

Welding of 40 cracks , 50 mm length each , on wood’s steel stays ( steel angle 50 X 50 X 5 ) .

Replacement of 3 handrail stanchions  , D 2” X 1100 mm .

Replacement of 10 steel angles 40 X 40 X 4 X 1300 mm of the truss .

Repair of 2 hanging plates .

006. GANGWAY No 40 – 20 m LENGTH

Manufacture & installation of one steel roller cover with 2 new HD hinges .

Manufacture & replacement of the roller shaft , D 40 X 1150 mm , with new steel bushes .

Replacement of old ball bearings from 4 transportation wheels with new bronze bushes ,

D 30 X D 62 X 70 mm .

Replacement of 8 handrail stanchions  , D 2” X 1100 mm , 2 pcs of horizontal rails & welding 12 cracks of stanchion bases .

007. GANGWAY No 315 – 15 m LENGTH

Replacement of the roller shaft , D 45 X 1150 , with 2 new steel bushes .

Manufacture & installation of one steel roller cover with 2 new HD hinges .

Cutting off the 2 transportation wheels , D 270 X 60 mm , from the roller side and rewelding them at new position at the truss’ edge  , replacement of their shaft , D 26 X 150 , with new bronze bushes .

Replacing of one stanchion of handrail .

Replacement of two rectangular hollow sections , 40 X 40 X 4 X 1500 mm of the truss .

Replacement of 10 wooden steps , 6 X 5 X 80 cm , from pine .

008. GANGWAY No 215 – 15 m LENGTH

Replacement of the roller shaft , D 45 X 1150 , with 2 new steel bushes , D 40 X D 60 X 50 mm , and restore the correct operation of the cover .

Manufacture & installation of one steel roller cover with 2 new HD hinges .

Cutting off the 2 transportation wheels , D 270 X 60 mm , from the roller side and rewelding them at new position at the truss’ edge , replacement of their shaft , D 26 X 150 , with new bronze bushes .

Replacement of 2 steel angles of the truss , 40 X 40 X 4 X 1500 mm and alignment of right handrail

in a length of 4 m .

009. GANGWAY No 43 – 20 m LENGTH

Replacement all the woods and steps as the old ones .

The total width of wood is 90 cm  , the steps are 50 pcs , 5 X 7 X 85 cm , from pine .

Welding of 44 cracks , 50 mm length each , on wood’s steel stays ( steel angle 50 X 50 X 5 ) .

Replacement of left & right handrails from steel pipe , D 2” .

The height of handrails must be 1100 mm .

Alignment of the truss , in a length of 3 m from the side of the hanging plates and replacement of

6 steel angles 40 X 40 X 4 X 1300 mm .

Manufacture of one bronze bush and fitting it on a transportation wheel .

Manufacture of 1 roller shaft D 40 X 1200 , with 2 new steel bushes .

C. PORT OF PPC ( 2 PCS )

001. GANGWAY No 54 – 25 m LENGTH

Welding of 20 cracks , 50 mm length each , on wood’s steel stays ( steel angle 50 X 50 X 5 ) .

Replacement of 3 steel angles , 40 X 40 X 4 X 1300 mm of the truss .

Manufacture & replacement of the roller shaft , D 45 X 1150 mm , with new steel bearings .

Repair of the roller cover .

Replacement of transportation wheel bushes .

The dimensions of wheels are D 200 X 50 .

002. GANGWAY No 50 – 25 m LENGTH

Replacement of all the woods and steps as the old ones .

The total width of wood is 90 cm  , the steps are 63 pcs , 5 X 7 X 85 cm , from pine .

Replacement of 2 pcs of steel angle 50 X 50 X 5 X 2000 mm of the handrails .

Manufacture and installation of a new steel roller cover with HD hinges .

Repair of the roller by replacement of shaft D 40 X 1150 mm with 2 new steel bushes .

Repair of the 4 transportation wheels , D 200 X 50 , by replacement of bronze bushes .

NOTES :

001. All above gangways must be prepared for painting by chipping hammer or any other mechanical

means .

002. The gangways will be painted same as the old colors , blue for the truss and yellow for the

handrails . RAL code will be provided upon assignment .

003. The coats for each color will be two : The first one with high-build epoxy and the second one

with aliphatic polyurethane paint .

004. All the old signs will be replaced by new , metallic , 2 pcs per gangway , dimensions

1 m X 1 m , in the center of the gangway .

On these signs will be written by metallic welded simple letters the identity of the gangway ,

e.g. PPA S.A. – No 35 , the max. load / inclination , e.g. 40000 Nt / 45° , the length , e.g. 25 m

and the max inclination , e.g. 30° .

The signs will be painted by two coats , as described at par. No 3 and the colors will be yellow for

the surface and black for the letters .

005. All transportation wheels , rollers and their covers must work properly and be secured as before .

006. The safety net that will be placed on each gangway should cover 90% of its length .

007. The contractor will be in charge of  load tests for each gangway , based on IMO regulation

MSC1 / cic. 1331 , section 5.3  and the issuance of the relevant inspection reports, which will be

submitted to the PPA.

In these reports will be described the N° of gangway , the material of it , the length & the width ,

the number and the distance between the steps , the height of handrails , the load test & the max.

deflection , any note and the result of test .

008.  All the bidders should inspect the gangways in order to have an integrated view of the works .

009. In the offer should be stated the cost & the delivery time per gangway as well as the total cost .

010. Any additional work will be carried out after the inspection and the approval of the PPA S.A. and

with a relevant offer.

PPA S.A. reserves the right to negotiate the offer or to request an offer from another company.

011. The repair works can be carried out either at the location of gangway or in the workshop of

repairer .

012. The order and quantity of the gangways to be repaired will be decided by PPA .