# ANNEX F: COMPLIANCE TABLE

|  | General Description | Minimum Requirements | Compliance Yes/No (to be filled in) | Ref. page of technical offer (to be filled in) |
| --- | --- | --- | --- | --- |
| Brand Model | Brand and Model | Blank |  |  |
| Type | Forklift type | Counterbalance including lifting bolt fixtures on counter weight frame |  |  |
| Capacity | Rated load capacity | ≥2.500 kg’s @ ≥500mm LC |  |  |
| Engine, transmission and electrical | Power type | Full electric, battery driven (time of fully recharging the batteries pack must be mentioned in order to be evaluated). Lithium-ion batteries is An obligatory requirement for fast charging. Side battery removal for battery swap (optional). |  |  |
|  | Drive system and axles | Of manufacturers proposal (detailed specifications to be mentioned) Steering type (Hydraulic or Electric) |  |  |
|  | Electrical system and components | Of manufacturers proposed brand (To include working LED lighting, buzzers, alarms, outdoor charges etc.) |  |  |
| Extra features | Fork side shift and independent fork positioning system. | Obligatory and of manufacturers proposal |  |  |
|  | Mast type | Free lift type of manufacturers proposal (able to load & unload a standard container cargo content) |  |  |
|  | Tyre brand and size | Of manufacturers proposal |  |  |
| Safety Features | Safety Systems | Load weight indicators. Stability control system. Automatic speed reduction system. Protection from dust and moisture (IP-rated components for outdoor use.) Overload system Gradeability control. |  |  |
| Extra features | Cabin type | Fully enclosed type with the appropriate cooling and heating system. |  |  |
|  | Seat type | Fully adjustable type for up to 150kg's SWL / well known brand. |  |  |
|  | Painting standards | Of manufactures proposal, with corrosion protection category for port enviroment (RAL code to be decided by PPA) |  |  |
| Other technical specifications | Fork mast tilt angle | Forward ≥ 5° - backward ≥ 10° degrees approx. |  |  |
|  | Travel speed | ≥ 15 km/h forward movement for full and empty (Speed selector switch will be favoured) |  |  |
|  | Brake type | Of manufactures proposal |  |  |
|  | Lifting height | ≥3000mm approx. |  |  |
| Dimensions | Fork load centre | ≥500mm |  |  |
|  | Total height including mast lowered | ≤2200mm approx. |  |  |
|  | Overall length (Without forks) | ≤3500mm approx. |  |  |
|  | Overall width | ≤1250mm approx. |  |  |
|  | Overall weight | Of manufacturers proposal |  |  |
|  | Fork sizes | ≥ 1070mm (42 inch) (*preferable, not an exclusion criterion) |  |  |
|  | Min. ground clearance | ≥110 mm approx. |  |  |
|  | General Description | Minimum Requirements |  | Proposal |
| Documentation, certification and spare parts | Documentation | Operators and Maintenance manuals in English, and Bill of Material (machine cards) to be provided in paper and CD. Spare parts book to be provided upon arrival of equipment. (Schematic drawings to be provided upon Offer). |  |  |
| Documentation, certification and spare parts | Certification | 1) To bear the CE marking, accompanied by the EC declaration of conformity |  |  |
|  |  | 2) Manufacturer to provide ISO 9001 certificate with Offer regarding the manufacture of the same kind of forklifts at the same manufacture site as those offered. |  |  |
|  |  | 3) Noise levels to be declared (according to Outdoor Noise Equipment applied EU Directive). |  |  |
| Spare parts | Spare part requirements | 1) Each forklift will be accompanient with one spare front complete wheel (rim, tyre etc) and one spare rear complete wheel (rim, tyre etc). for each forklift. |  |  |
|  |  | 2) Each forklift will be accompanient with one full set of filters & spare parts for the maintenance intervals of the first 500 operating hours, according to manufacturer preventative maintenance plan. |  |  |
|  |  | 3) One (1) extra charger (spare) and one (1) extra battery pack (spare) should be delivered in total, accompanied with all proper equipment for charging of forklift. |  |  |
|  |  | 4) One (1) complete operator's seat (spare) accompanied with all proper equipment for fitting on forklift |  |  |
|  |  | 5) One (1) complete set of lights (spare) i.e. front & rear working lights mast lights, hazard lights, beacon, etc. |  |  |
| Delivery time | Delivery time | Up to seven (7) months ready to operate at Cruise Terminal of P.P.A. S.A. |  |  |
| Warranty time | Warranty | At least three (3) year for structure and parts (Excluding wear and tear parts). Availability of parts and technicians during warranty time to be stated. Diagnostic systems for support to be stated |  |  |